UPDATED USE CASE SCENARIOS

Use Case Scenario means that what can a user to with a system. Our vending machine controller on FPGA is designed to simulate the interactions between a user and a vending machine. In the beginning of our projects, we have come up with several requirements and I have formulated the use case scenarios based on these requirements.

<State: IDLE>

1. When a user turns on the FPGA board, it remains in the state IDLE and the first LED (0) will be lighted up to indicate the state status.

2. When the button in the middle (BTNC) is pressed, then the state goes from IDLE to DISPLAY.

<State: DISPLAY>

3. The user should see the second LED (1) is lighted up.

4. In this state DISPLAY, a user can select one of the three items (Cola, Beer, Water) by turning on one of the three switches (SW (0), SW (1), SW (2)), and the user can see the product information on the 8 digits of seven segment display. To be more specific, the user should be able to see the item name on the left four digits of seven segment display and the item price on the right four digits of seven segment display.

5. If a switch is up and the BTNC is pressed, then the state goes to the INSERT state.

<State: INSERT>

6. The user should see the third LED (2) is lighted up.

7. Also, the user should be able to see the price of the selected items on the left four digits of 7 segment display and 00.00 on the right four digits of the 7 segment display.

8. When one of the four buttons is pressed (BTNU, BTNL, BTNR, BTND, each respectively 10 cents, 20 cents, 50 cents, and 1 euro), the user should see the value of coin inserted. The user should be able to see the value getting added up and displayed on the right four digits of the seven segment display.

9. When the inserted coins are bigger or equal to the price of the items, the user can press the BTNC to go to the next state, DISPENSE.

<State: DISPENSE>

10. The user should see the last LED (3) is lighted up.

11. The user should see the change values to be returned to the user on the right four digits of the 7 segment displays, which is the value of inserted coins minus the price of the item.

12. The user can reset the system back to the IDLE for the next user by turning the switch off and pressing the BTNC.