1. Power_Supply Sheet

Purpose:
Provides stable voltage supply (3.3V and 5V) to all digital components and logic.

Key Components:

- LM7805 → 5V regulator

- LM317 → Adjustable regulator set to 3.3V

- Barrel jack → External DC input

- PWR_FLAG symbols → Inform ERC that power is intentionally supplied

- Capacitors → For voltage smoothing and stability

This sheet generates regulated 5V and 3.3V required by the FPGA and other parts. The LM317 outputs 3.3V using R2 and R4. PWR_FLAGs ensure KiCad knows power is being supplied properly.

Coin_Buttons Sheet

Purpose:
Handles the physical pushbuttons simulating coin input (10c, 20c, 50c, 1€, Confirm).

Key Components:

- SW4 to SW8 → Represent coin inputs

- Pull-down resistors (330Ω) → Prevent floating input

- Global labels (BTNU, BTNL, BTNR, BTND, BTNC) → Connect to FPGA_IO logic

“Each button simulates a different coin value. Resistors avoid floating states, and labels allow signals to connect across sheets. These inputs drive state changes and insert coins in the FSM.”

3. Display_Interface Sheet

Purpose:
Controls a CA56-12EWA 4-digit 7-segment display to show product names, prices, and change.

Key Components:

- CA56-12EWA → 4-digit display

- PNP Transistors (Q1–Q4) → Control digit activation

- Base resistors (1kΩ) → Protect FPGA pins

- Segment lines (SEG_A to SEG_G, SEG_DP) → Shared by all digits

- Digit selects (AN0 to AN3) → Control which digit is active

“This circuit displays characters by multiplexing the 4-digit display. Segments show product name and price. PNP transistors let us turn on one digit at a time while segments decide what to show.”

4. FPGA_IO Sheet

Purpose:
Central pin mapping between FPGA and all external modules (buttons, display, LEDs, switches).

Key Components:

- FPGA header symbol

- Labels like SEG_A, AN0, BTNU, SW0, LED0, etc.

- Global signals – Connects coin buttons, switches, LEDs, and 7-segment segments

“This sheet maps all input/output signals from the FPGA board. It links the FSM logic with the physical interface—buttons, display, and switches.”

5. Root Sheet (Top-Level Integration)

Purpose:
Integrates all 4 sheets together using hierarchical sheet blocks and global labels.

“This top sheet connects all subsystems. Labels are routed between blocks so data flows from coin buttons to FPGA logic and to the display for real-time feedback.”

Logic

- User powers the board → Power supply stabilizes 3.3V and 5V.

- FPGA enters IDLE → State indicated by LED0.

- Press Confirm (BTNC) → transitions to DISPLAY mode.

- Use switches (SW0–SW2) to select item.

- Insert coins (via BTNU/BTNL/BTNR/BTND) → shown on 7-segment display.

- When balance is enough, FSM transitions to DISPENSE.

- Change is displayed; system resets to IDLE.